[ CLIENT COMPANY NAME ] CONTROLLED DOCUMENT SYSTEM

Operational Risk Control System

Working at Heights

Safe Work Method Statement (SWMS) for high-risk construction work - critical controls, field verification and hold points. Prepared, kept on site and followed by all workers; tailor to your site and authorise before commencing.

| Document No. | SWMS-WAH-001 | Revision | A |
| --- | --- | --- | --- |
| Version | 1.0 | Issue date |  |
| Review date |  | Category | Operational Risk Control System |
| Risk class | High-Risk Construction Work | Status | CONTROLLED |
| Jurisdiction | National (model WHS laws) | WHS regulator | Safe Work Australia (model WHS laws) |

CONTROLLED DOCUMENT - Uncontrolled when printed. This document must be tailored to your project and approved by a competent person before use.

Scan to verify  -  SWMS-WAH-001  |  Rev A  |  Version 1.0  |  CONTROLLED

OBRAS OPERATING STATUS - GREEN / AMBER / RED

| GREEN - PROCEED | AMBER - REVIEW | RED - DO NOT PROCEED |
| --- | --- | --- |
| Current approved revision. All controls in place, competencies verified and sign-offs complete. Work may proceed. | Pending approval, near expiry, or a control needs checking. Verify and clear it before work starts. | Expired, superseded, unsigned, or a critical control has failed. Stop - do not proceed until cleared. |

This document is GREEN only when it is the current revision AND every control, competency and sign-off is complete. Scan the QR to verify its live status at the point of work.

Safe Work Method Statement (SWMS)

Working at Heights

| SWMS No. | SWMS-WAH-001 | Version | 1.0 | Date issued | dd/mm/yyyy |
| --- | --- | --- | --- | --- | --- |
| Company / PCBU | your company | Prepared by | name | Next review | dd/mm/yyyy |
| Project / Site | site address | Principal contractor | name | Approved by | name / position |

| FEEDS | RECEIVES FROM | SUPPORTS |
| --- | --- | --- |
| SWMS Register; SWMS Approval Register | Site induction; VOC competency | Daily Readiness Dashboard (go / no-go) |

HIGH-RISK CONSTRUCTION WORK (HRCW)

This SWMS covers High-Risk Construction Work under the WHS Regulation - specifically work where there is a risk of a person falling more than 2 metres - including work on roofs, scaffolds, elevating work platforms (EWPs), ladders, formwork and near unprotected edges, penetrations and fragile surfaces. A SWMS must be prepared before this work starts, kept on site, and followed by all workers.

SCOPE OF WORK

| Item | Details - complete for your site |
| --- | --- |
| Describe the work this SWMS covers (what, where and how) |  |
| Work area / location on site (level, area, grid) |  |
| Start date and expected duration |  |
| Other High-Risk Construction Work involved (if any) |  |

LEGISLATION, CODES & STANDARDS

This SWMS aligns with the work health and safety laws of the relevant Australian jurisdiction. Apply the Act and Regulations for your state or territory:

| Jurisdiction | Principal WHS / OHS Act & Regulations |
| --- | --- |
| Commonwealth, NSW, QLD & ACT | Work Health and Safety Act 2011 & WHS Regulations 2011 |
| Northern Territory | WHS (National Uniform Legislation) Act 2011 & WHS (NUL) Regulations |
| South Australia & Tasmania | Work Health and Safety Act 2012 & WHS Regulations 2012 |
| Victoria | Occupational Health and Safety Act 2004 & OHS Regulations 2017 |
| Western Australia | Work Health and Safety Act 2020 & WHS (General) Regulations 2022 |

Codes of Practice (Safe Work Australia):

- Construction Work

- How to Manage Work Health and Safety Risks

- Managing the Risk of Falls at Workplaces

- Managing the Risk of Plant in the Workplace

- Managing Noise and Preventing Hearing Loss at Work

- Hazardous Manual Tasks

- Managing Risks of Hazardous Chemicals in the Workplace

- WHS Consultation, Cooperation and Coordination

Australian Standards:

- Code of Practice: Managing the risk of falls at workplaces

- AS/NZS 1891 - Industrial fall-arrest systems and devices

- AS/NZS 1576 - Scaffolding

- AS/NZS 1892 - Portable ladders

- AS 2550.10 / AS 2550.1 - Cranes, hoists and winches (elevating work platforms)

COMPETENCY, TRAINING & PLANT

- All workers hold a current Construction Induction Card (White Card)

- Workers are trained and competent in working at heights; training records held on site

- EWP operators hold the relevant High Risk Work Licence (WP for boom > 11 m) and/or a Yellow Card / VOC for the specific unit

- Scaffold above 4 m is erected, altered and dismantled only by a licensed scaffolder

- Anyone using a fall-arrest harness is trained in its fitting, inspection and the rescue plan

- A trained rescuer and rescue equipment are on site and available before work starts

Plant & equipment used:                                                                 EWP (scissor / boom lift), mobile and fixed scaffold, ladders, fall-arrest harness and energy-absorbing lanyards, certified anchor points, guardrails and edge protection, safety mesh, tool lanyards

PERSONAL PROTECTIVE EQUIPMENT (PPE) - MANDATORY ON SITE

| Hard hat | Hi-vis | Safety boots | Safety glasses | Gloves | Fall-arrest harness |
| --- | --- | --- | --- | --- | --- |

HIERARCHY OF CONTROL (always apply controls from the top down)

| Rank | Control level | How it works | Examples |
| --- | --- | --- | --- |
| 1 | Elimination | Remove the hazard completely - the most effective control | Do the task at ground level; design the hazard out |
| 2 | Substitution | Replace it with something less hazardous | Use a less hazardous material, method or item of plant |
| 3 | Isolation | Separate people from the hazard | Exclusion zones, barricades, guarding, isolation / lock-out |
| 4 | Engineering | Use engineering controls | Guards, RCDs, mechanical lifting aids, ventilation, edge protection |
| 5 | Administrative | Change the way people work | SWMS, procedures, training, permits, signage, supervision |
| 6 | PPE | Protect the worker - the last line of defence | Hard hat, hi-vis, boots, gloves, eye & hearing protection, RPE, fall arrest |

JOB STEP RISK ASSESSMENT

| # | Job step / activity | Hazards / what could go wrong | Initial | Control measures (hierarchy of control) | Residual | Responsible |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Plan the task and confirm the method | Wrong access method chosen; fall hazards not identified; workers not briefed | High | ELIMINATE first - do the work from the ground or off a solid structure where possible (prefabricate, use extension tools). Where work at height is unavoidable, select the highest-order access method (EWP or scaffold before ladders). Confirm the SWMS on site and brief all workers before work starts. | Low | name |
| 2 | Assess the work area and establish exclusion zones | Fall through penetrations / fragile roof; objects falling on people below; public or other trades entering the drop zone | High | Identify and cover or guard all penetrations, voids and skylights. Mark fragile surfaces and never walk on them. Set a hard-barricaded exclusion zone below the work with signage; control access so no one is under the work. | Low | name |
| 3 | Set up edge protection / guardrails | Fall from an unprotected edge | Extreme | Install compliant guardrails, mid-rails and toe-boards to all open edges as the primary (passive) fall-prevention control. Inspect for completeness and security before use. Do not remove edge protection while work continues nearby. | Low | name |
| 4 | Set up and inspect the EWP / scaffold | Overturn, entrapment, structural failure, electrocution from overhead lines | High | Complete the pre-start / log book on the EWP; check ground bearing, slope and outriggers. Maintain safe approach distances from overhead powerlines (use a spotter). Confirm scaffold has a current green scaftag and full planks, guardrails and access before use. | Low | name |
| 5 | Fit and connect fall-arrest harness and anchor points | Fall arrest fails; worker not connected; anchor pulls out; suspension trauma | Extreme | Where a fall risk remains, use a fall-arrest system as the last line of defence. Inspect harness, lanyard and connectors before each use and tag out any defect. Connect only to a certified anchor rated for fall arrest. Keep the lanyard short to limit free-fall and ensure adequate clearance below. | Low | name |
| 6 | Access the elevated work position | Fall while climbing; ladder slips or is over-reached | High | Use ladders only for short-duration, light work; maintain three points of contact, 4:1 angle, and tie off / foot the ladder. Do not over-reach - reposition instead. Enter EWP baskets only when grounded and exit only at the work level. | Low | name |
| 7 | Carry out the work at height | Fall; dropped tools and materials strike people below; loss of balance | High | Stay within the guardrails / basket and remain connected. Use tool lanyards and closed containers; never throw materials. Keep the work area tidy. Do not lean out, climb on rails, or stand on mid-rails. | Low | name |
| 8 | Monitor weather and changing conditions | Wind, rain, heat or storm increases fall and electrocution risk; surfaces become slippery | High | Check the forecast and stop EWP / roof / scaffold work in high wind, lightning or wet conditions per the manufacturer and site limits. Re-assess after any change to the task, plant, site, weather or crew, and re-authorise before resuming. | Low | name |
| 9 | Respond to an emergency / rescue from height | Worker suspended in harness (suspension trauma); injury at height; delayed rescue | Extreme | Have a documented rescue plan and rescue equipment ready before work starts, with a trained rescuer on site. Aim to release a suspended worker within minutes. Call 000, make the area safe, and do not attempt a rescue that puts the rescuer at risk. | Low | name |
| 10 | Pack down and clear the work area | Falls during dismantling; leaving open edges, penetrations or trip hazards | Medium | Dismantle in the reverse order of setup; keep edge protection in place until last. Reinstate permanent guarding or covers over penetrations. Remove waste, coil leads and leave the area safe for the next crew. | Low | name |

RISK MATRIX & RATING KEY (Risk = Likelihood x Consequence)

| Likelihood \ Consequence | Insignificant (1) | Minor (2) | Moderate (3) | Major (4) | Catastrophic (5) |
| --- | --- | --- | --- | --- | --- |
| Almost certain (5) | 5 | 10 | 15 | 20 | 25 |
| Likely (4) | 4 | 8 | 12 | 16 | 20 |
| Possible (3) | 3 | 6 | 9 | 12 | 15 |
| Unlikely (2) | 2 | 4 | 6 | 8 | 10 |
| Rare (1) | 1 | 2 | 3 | 4 | 5 |

| LOW (1-3) | MEDIUM (4-6) | HIGH (8-12) | EXTREME (15-25) |
| --- | --- | --- | --- |

EMERGENCY & RESPONSE

| Site emergency no. | phone | First aider(s) | name(s) |
| --- | --- | --- | --- |
| Nearest hospital | name / address | Assembly point | location |

CRITICAL CONTROLS, FIELD VERIFICATION & HOLD POINTS

These controls are mandatory and must be verified in the field before and during the work. Work must not pass a hold point until it is inspected and released by the responsible person.

| # | Critical control (must be in place) | Verified Y/N | Verified by | Hold point - work stops until released |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

| Authorised to commence by | name / position | Signature / date |  |
| --- | --- | --- | --- |

STOP WORK TRIGGERS (stop immediately, make the area safe, and re-authorise before resuming)

- A critical control is missing, fails, or is bypassed

- Site conditions change beyond this SWMS (weather, ground, services, access, adjacent work)

- An incident, injury, near miss or dangerous occurrence happens

- An unlicensed, untrained or unauthorised person enters the work zone

- Plant or equipment shows a defect, alarm or fault, or is outside its inspection date

- Any worker is unsure it is safe to continue - anyone on site can call a stop

ONGOING MONITORING & VERIFICATION REQUIREMENTS

Controls must be monitored throughout the work - not just at the start. Record what is checked, how, how often and by whom.

| What is monitored | Method | Frequency | Responsible |
| --- | --- | --- | --- |

WORKER SIGN-ON (I have read, understood and agree to follow this SWMS)

| # | Name | Company | Signature | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

SUPERVISOR VERIFICATION & SIGN-OFF

| SWMS reviewed on site by (supervisor) | name / signature | Date | dd/mm/yyyy |
| --- | --- | --- | --- |
| Controls confirmed in place | [ ] Yes [ ] No | HRW licences / VOCs verified | [ ] Yes [ ] N/A |
| Workers consulted & signed on | [ ] Yes | Safe to proceed | [ ] Yes [ ] No |

SWMS REVIEW

Review this SWMS if the work, plant, site conditions or workers change, after any incident or near miss, or at the scheduled review date.

| Review date | Reviewed by | Reason / changes made | Signature |
| --- | --- | --- | --- |

REVISION REGISTER

| Rev | Date | Description of change | Approved by |
| --- | --- | --- | --- |
| A |  | Initial controlled release | [ CLIENT COMPANY NAME ] |

IMPORTANT - READ BEFORE USE This document is provided by OBRAS ONSITE as a starting-point template only. You are responsible for ensuring it is appropriate for your specific workplace, tasks, plant and site conditions. It must be reviewed, edited to suit the actual site, and signed off by a competent person before use. This template does not constitute professional WHS advice and OBRAS ONSITE accepts no liability arising from its use. We strongly recommend your completed document is reviewed by a qualified WHS professional or competent person. (c) 2026 OBRAS ONSITE. All Rights Reserved. OBRAS ONSITE, the OBRAS ONSITE Operating System, OBRAS ONSITE QR Gateway and associated methodologies, control systems and documentation structures are proprietary OBRAS ONSITE intellectual property; unauthorised reproduction, resale, distribution or commercial use is prohibited.